CPC CORPORATION, TAIWAN

SOLVENT & CHEMICAL BUSINESS

Xylene Nitration Grade (D730)

Date: 2007/03/03  Version 3.0

Product No. 203-P04285

Specification      Test Method

CNS      ASTM

Appearance            	Visual

Sp.Gr. at 15℃/15℃ .                	0.865~0.875  	14474     D4052

Density, 20℃, g/cm3               	0.862~0.872    14474     D4052

Color, Pt-Co Scale,                   	max. 20   	2755 K6254 D1209

Distillation Range,℃                	max. 5    	2756 K6255 D850

I.B.P. ,                         	min. 137

Dry Point, ℃,                  	max. 143

Nonaromatics, vol%             	max. 4.0   	2761 K6260  D2360

Acidity                             	2758 K6257   D847

Acid Wash Color                     	max.No.6  	2757 K6256   D848

Copper Corrosion, 1/2hr. at 100℃               	2760 K6259   D849

Hydrogen Sulfide and Sulfur Dioxide（H2S,SO2 ）   	2759 K6258  D853