CPC CORPORATION, TAIWAN

SOLVENT & CHEMICAL BUSINESS

NORMAL PARAFFIN (D907)

Date：2010/02/23，Version 2.0

Product No. : 203P0150700 、203P 0150900、116S 0150800

∣2.1

| Item | Specification | Test Method （ASTM） |
| --- | --- | --- |
| Appearance | Clear Liquid |  |
| Color Saybolt | 29 |  |
| Bromine mg/100g | 25 Max. | D1492 |
| Sulfur ppm | 5 Max. | D5453 |
| Aromatic wt % | 0.25 Max. | UOP495 |
| Purity wt % | 98.5 Min. |  |
| Carbon Distribution wt % |  | Gas Chromatography |
| C9 and lighter wt % | 0.5 Max. |  |
| C10 wt % | 10 Max. |  |
| C11 wt % | As report |  |
| C12 wt % | As report |  |
| C13 wt % | 15 Min. |  |
| C14 and heavier wt % | 1 Max. |  |
| MW | 164 Min. |  |

Note：

Use for the raw materials of manufacturing biodegradable linear alkyl benzene. That is raw materials of surfactant detergent. It also uses for manufacturing Chlorinated Paraffin which is a kind of plasticizer of PVC. It is raw materials of green lamp-oil and less toxic washing solvent.

∣1.1