CPC CORPORATION, TAIWAN

SOLVENT & CHEMICAL BUSINESS

Methyl Alcohol (D905)

Date:2013/02/25，Version 2.1

Product No. : 203P0500100

∣2.1

| Item | Specification | Test Method（ASTM） |
| --- | --- | --- |
| *Purity | Min. 99.85 wt % | E346/IMPCA 001 |
| Specific Gravity at 20/20°C | 0.792-0.793 | E346/D891/D4052 |
| *Color | Max. No. 5 | E346/D1209 |
| *Distillation Range | Max 1.0°C to include 64.6°C ± 0.1°C | E346/D1078 |
| *Water | Max. 0.1 wt % | ## E203/E1064 |
| Odor | Characteristic, Non residual | D1296 |
| Non-volatile Matter | Max. 5 mg/100ml | D1353 |
| Acetone | Max. 0.003 wt % | E346/IMPCA 001 |
| Hydrocarbons | Pass Test | E346/D1722 |
| Carbonizable Substances | Max. No. 50 | E346/D1209 |
| Acidity | Max. 0.003 wt % | E346/D1613 |
| Potassium Permanganate time | Min. 50 minutes | E346/D1363 |
| Chloride | Max. 1 ppm | D5085/IMPCA002 |
| Alkalinity as NH3 | Max. 0.003 wt % | D1614 |
| Ethanol | Max. 50 ppm | E346/IMPCA 001 |
| Sulfur | Max. 1 ppm | D4045/D3691 |

Note：

Use for manufacturing formaldehyde, formic acid, inorganic acid ester, organic acid ester, refrigerant, cleaner, perfume, photo-film… etc. Methyl Alcohol, molecular formula is CH3OH. In early years it comes from separated distillate of wood dry distillation.

Methyl Alcohol is an inflammable, colorless, and toxic liquid.

Its melting point is -93.9℃, boiling point is 65℃, related density is 0.7914(20/4℃). It can dissolve in water, ethanol, ether, acetone, benzene and other organic solvents.